ROV VEHICLE

Swift-07 ROV System-150hp

Depth Rating				        	     1,000 meters

Total Power Rating				          113kw/150hp

### Dimensions & Weight

Length						                  2,500 mm

Width                      	                       1,450 mm

Height						                  1,800 mm

Weight In-air                                  	  2,700 kgs

Performance

Forward				           300 kgf	   3.5knots

Lateral				                265 kgf	   2.5knots

Vertical				           225 kgf	   2.0 knots

Turning Rate     		                   40°/s @ rated depth

Standard Power Pack 150hp

MTQ 113kw/150hp shaft output power, 3 phases, 4 poles,

3KV motor, oil filled and compensated.

Vickers piston pump, output 220lpm @ 200bar

Payload (including manipulators)

Payload of Standard Vehicle		                   150 kgs

Optional Additional Payload		                    50 kgs

Through Frame Lift				            2,500 kgs

Propulsion System

7 MTQ 350mm diameter 12.5kw/16.5hp hydraulic thrusters

individually servo valve controlled

Lateral			4 x 350 mm Ø thrusters

Vertical	     3 x 350 mm Ø thrusters

Video & Telemetry System (Standard)

Transmission via fiber optics and copper

Fiber Optic, 3 video + 6 RS 232 / 485 channels

Copper, 3 Video + 8 Data Lines

Real-time Video Channels, 3 on Coax / 3 on Fiber

Pan & Tilt

MTQ Pan & Tilt Unit	                                   1

Pan	                                               ±180°

Tilt	                                               ±180°

Second Pan & Tilt optional

Cameras

OE 1323 D SIT Camera	   1

OE 1360 Color Camera	   1

Still Camera with Flasher (optional)

Digital Still Camera & Flasher (optional)

Control & Navigation

Flux Gate Compass and Integrated Rate Gyro System       1

TritechSeaKing DFS Sonar system	                        1

MTQ Depth Transducer System	                        1

Power Packs & Tooling Interfaces (optional fit)

MTQ Hydraulic Power Pack 125hp

MTQ Tooling Power Pack 50hp

Schilling 7-Function Titan T3 Manipulator

Schilling 5-Function RigMaster Grabber Arm

Ragan Latch Override Tool

Guide Wire Cutter

75mm diameter Wire Rope Cutter

API Standard Hot Stabs

API Torque Tools

supplied on request.

UMBILICAL

The standard Aramid umbilical is 41mm in diameter

and has the following cores:

17 x 4mmsq 3kv Rated Power and Earth Cores

4x 75 ohm Coaxial

4 x Screened Twisted Quads

4 Single + 2 Multi Mode Fibers in a common loose tube

Standard length 650 meters, or 1200 meters optional

DECK CABLE

30m Deck cable from Control Van to Umbilical Winch

Swift-07 ROV System-150hp

### CONTROL CONTAINER (A60, ZONE2 OPTIONAL)

Length		                                       6,000 mm

Width		                                       2,438 mm

Height	           	                            2,591 mm

Shipping Weight		                            7,000 kgs

WORKSHOP CONTAINER (A60, ZONE2OPTIONAL)

Length		                                       6,000 mm

Width		                                       2,440 mm

Height 		                                  2,774 mm

Shipping Weight		                           12,000 kgs

Complete inventory of system spares

Mechanical and electrics tools and test equipment

HANDLING SYSTEM (A60, ZONE 2 OPTIONAL)

Exd II BT4 Explosion-Proof Level

Hydraulic Marine Crane

SQ10ZA2M Model Marine Crane 	                   1

Length 		                                  3,000 mm

Width 		                                       1,600 mm

Height 		                                  2,600 mm

Shipping Weight 		                             5,000 kgs

Arm maximum reach 		                        6,000 mm

Maximum lift capacity 		                             10 t

Lift capacity @ full extension 	                        3,200 kgs

Power Rating		                                 30 kw

Umbilical Winch

243JC Model Marine Winch 	                            1

Length 		                               2,500 mm

Width 		                               2,450 mm

Height 		                               2,300 mm

Shipping Weight 		                          6,000 kgs

Line Pull 		                                1,300 kgf

Line Speed 		                         0~30 m/min

Drum Capacity (Ф 35.5mm cable)                  ≥800 m

Hydraulic Power Pack for Handling System

Length			                          6,000 mm

Width			                               2,440 mm

Height 		                               2,774 mm

Shipping Weight			                    12,000 kgs

SERVICERS

Power (total) 380/440V, 300A, 50/60Hz

Air (LP)	optional for Zone II only

GENERATOR (OPTIONAL)

Capacity, 380/440V, 3phases, 50/60Hz

350kVA, 500A.

Depth Rating				        	     1,000 meters

Total Power Rating				           94kw/125hp

### Dimensions & Weight

Length						                  2,500 mm

Width                      	                       1,450 mm

Height						                  1,800 mm

Weight In-air                                  	  2,700 kgs

Performance

Forward				           300 kgf	   3.5knots

Lateral				                265 kgf	   2.5knots

Vertical				           225 kgf	   2.0 knots

Turning Rate     		                   40°/s @ rated depth

Standard Power Pack 150hp

MTQ 94kw/125hp shaft output power, 3 phases, 4 poles,

3KV motor, oil filled and compensated.

Vickers piston pump, output 220lpm @ 200bar

Payload (including manipulators)

Payload of Standard Vehicle		                   150 kgs

Optional Additional Payload		                    50 kgs

Through Frame Lift				            2,500 kgs

Propulsion System

7 MTQ 350mm diameter 12.5kw/16.5hp hydraulic thrusters

individually servo valve controlled

Lateral			4 x 350 mm Ø thrusters

Vertical	     3 x 350 mm Ø thrusters

Video & Telemetry System (Standard)

Transmission via fiber optics and copper

Fiber Optic, 3 video + 6 RS 232 / 485 channels

Copper, 3 Video + 8 Data Lines

Real-time Video Channels, 3 on Coax / 3 on Fiber

Pan & Tilt

MTQ Pan & Tilt Unit	                                   1

Pan	                                               ±180°

Tilt	                                               ±180°

Second Pan & Tilt optional

Cameras

OE 1323 D SIT Camera	   1

OE 1360 Color Camera	   1

Still Camera with Flasher (optional)

Digital Still Camera & Flasher (optional)

Swift-01 ROV System-125hp

Control & Navigation

Flux Gate Compass and Integrated Rate Gyro System       1

TritechSeaKing DFS Sonar system	                        1

MTQ Depth Transducer System	                        1

Power Packs & Tooling Interfaces (optional fit)

MTQ Hydraulic Power Pack 125hp

MTQ Tooling Power Pack 50hp

Schilling 7-Function Titan T3 Manipulator

Schilling 5-Function RigMaster Grabber Arm

Ragan Latch Override Tool

Guide Wire Cutter

75mm diameter Wire Rope Cutter

API Standard Hot Stabs

API Torque Tools

supplied on request.

UMBILICAL

The standard Aramid umbilical is 41mm in diameter

and has the following cores:

17 x 4mmsq 3kv Rated Power and Earth Cores

4x 75 ohm Coaxial

4 x Screened Twisted Quads

4 Single + 2 Multi Mode Fibers in a common loose tube

Standard length 650 meters, or 1200 meters optional

DECK CABLE

30m Deck cable from Control Van to Umbilical Winch

Swift-01 ROV System-125hp

### CONTROL CONTAINER (A60, ZONE2 OPTIONAL)

Length		                                      6,000 mm

Width		                                      2,438 mm

Height		                                      2,591 mm

Shipping Weight		                           7,000 kgs

WORKSHOP CONTAINER (A60, ZONE2OPTIONAL)

Length		                                      6,000 mm

Width		                                      2,440 mm

Height 		                                 2,774 mm

Shipping Weight		                          12,000 kgs

Complete inventory of system spares

Mechanical and electrics tools and test equipment

HANDLING SYSTEM (A60, ZONE 2 OPTIONAL)

Exd II BT4 Explosion-Proof Level

Hydraulic Marine Crane

SQ10ZA2M Model Marine Crane 	                      1

Length 		                                 3,000 mm

Width 		                                      1,600 mm

Height 		                                 2,600 mm

Shipping Weight 		                            5,000 kgs

Arm maximum reach 		                       6,000 mm

Maximum lift capacity 		                           10 t

Lift capacity @ full extension                	       3,200 kgs

Power Rating		                               30 kw

Umbilical Winch

243JC Model Marine Winch 	                             1

Length 		                                2,750 mm

Width 		                                2,440 mm

Height 		                                2,590 mm

Shipping Weight 		                           6,000 kgs

Line Pull 		                                 1,300 kgf

Line Speed 		                          0~30 m/min

Drum Capacity (Ф 35.5mm cable)                   ≥800 m

Hydraulic Power Pack for Handling System

Length			                           6,000 mm

Width			                                2,440 mm

Height 		                                2,774 mm

Shipping Weight			                     12,000 kgs

SERVICERS

Power (total) 380/440V, 300A, 50/60Hz

Air (LP)	optional for Zone II only

GENERATOR (OPTIONAL)

Capacity, 380/440V, 3phases, 50/60Hz

350kVA, 500A.

MAJAVE Observation class ROV

### Type

300 meters depth rating

Manufacturer

MTQSubsea Technology Pte Ltd

Depth Rating

Standard Depth rating                       300 meters

Propulsion system

5 SPE-75 brushless direct current propellers

Propulsion mode Vernier Thruster

Dimensions & Weight

Length					         1000 mm

Width                	               600 mm

Height					          500 mm

Weight In-air(self-weight)	                 85 kg

Length of umbilical	                  350 meters

Performance

Forward					           63kgf

Lateral					           63kgf

Vertical				             	26kgf

Power requirement

3.6 KW, 90V-230Valternating current