DIV

DIV Diving Engineering Co., Ltd

SAT6 300msw/12 man

Saturation Diving System

Specifications                               DIV

DIV Diving Engineering Co., Ltd

SAT6 300msw/12 man Saturation Diving  System

### MAIN PARTICULARS

Year built                                                                                 2014

Builder                                                                        IUC / Unique

Class Society                                                                           ABS

Testing/Certification                                            IMCA D018/D024

Standard

### COMPONENTS

Weight     Dimensions (L*W*H)

DDC1 with TUP                                 32.16t            9.14*3.30*3.56

HRC                                                  19.44t            7.55*5.64*3.09

DDC2                                                18.16t            4.95*3.05*4.44

Bell                                                    30.71t            4.80*4.60*6.71

Sat/Air Diving Control Van                13.60t            7.62*3.05*2.84

Life Support Machine Van                 7.50t             6.07*2.91*2.59

Gas Reclaim Van                               8.20t             6.07*2.91*2.59

LARS                                                 38.00t           7.01*3.55*5.26

Emergency Generator                       6.30t             3.07*2.44*2.59

### SERVICES REQUIRED

Electric Power - System                                              480V, 60HZ

Electric Power – Hot water machine                           480V, 60HZ

Emergency generator                                       250kV 480V,60HZ

Seawater                                               217Ipm / 50 psi minimum

Fresh Water                                          Domestic potable@30 psi

### LIVING ACCOMMODATION CHAMBERS

DDC1                                                                                   3 man

DDC2                                                                                   3 man

HRC                                                                                     6 man

### DIVING BELL

Capacity men                                                                              3

Diver Lockout Capability                                                             2

Diver Hat Mounted Camera Systems                                         2

Volume                                                                             4.56 m³

Mating to DDC                                                                       Side

Buoyancy                                                                        Negative

### RECLAIM SYSTEM

Type                                                         Lexmar LezGas System

Compressor                                                                     CompAir

Bell Capacity                                                         2 Diver Reclaim

Dive Control System                                                             Divex

### HYPE RBARIC EVACUATION SYSTEM

Type                                                      Side-off/Float-off HRC

Launch System                                   Self Launching System

Propulsion                                                                  Tow only

Capacity men                                                                        12

Control Van                                            Dedicated w/umbilical

### UMBILICAL

Main Bell                                                                          350m

Lockout Diver1                                                                   45m

Lockout Diver2                                                                   45m

Bell men                                                                             55m

### LIFE SUPPORT MACHINERY

Diver Hot water Unit                                               CDI Supply

CUM Units                                                                         UTS

Scrubbers                                                                    DYDRA

Specifications                               DIV

DIV Diving Engineering Co., Ltd

HAI YANG SHI YOU 289

### GENERAL DESCRIPTION

Type                                  MP Subsea Construction Vessel

Design                                                         STX OSCV 11L

Built by                                 NB 776 STX Søviknes Norway

Accommodation                                                100 persons

Gross Tonnage                                                             8922

Net Tonnage                                                                 2677

IMO No                                                                   9657882

Call Sign                                                                    BFBC8

Area of  Operation                                               Worldwide

### Classification

1A1,E0,DYNPOS-AUTR(IMO II),DK(+),

CRANE,HELDK,ICE-1B.DEICE.CLEAN DESIGN,

NAUT-OSV,COMF-V(3),SPS

### Main Dimensions

Overall length                                                            120.8m

Length between p.p                                                  111.2m

Breadth molded                                                          22.0m

Depth main deck                                                           9.0m

Draft(max)                                                                     6.6m

### Deck Equipment

2×Windlass/Mooring winches fwd.Hatlapa

2×Mooring winches aft-Hatlapa

2×Provision cranes-Cargotech/MacGregor 3t

### Deck Space

Deck Material                                                      Steel/wood

Moonpool Work                                                     7.2×7.2m

Moonpoo ROV                                                       4.8×5.5m

Main Deck Area                                             1300㎡10t/㎡

### DP System

DPII                                                Kongsberg SDP 21(DP2)

Ref.Systems                                                   Gyrocompass

3×STD22 Anschutz

2×DGPS(Seatex132&232)

1×Radius 1000

2×Hipap 500

2×Seapath 300

### NAV/Com

X-Band / S-Band radar

1×Furuno FE-700 Echo sounder

2×Telchart Dual TECDIS T-2138A

1×Sailor 6110 Sat C

1×Fleet Broadband 250 Sat phone

1×C band VSAT

### Power & Propulsion

Main          4×Wartsila 6L32 each 2880 kW(total 11 520 kW)

SCR System

Em.Gen                       1×Nogva Scania DI 1262m 285 kW

### Thruster

Stern  2×RR Contaz 35 (Ice strenghtened) each 3000 kW

Bow   2×RR TT2650 each 1900 kW

1×RR TCN92M each 1500 kW

### Rescue

1×Weedo 700-222 hp waterjet

1×Midget 500 mkII-50hp outboard

2×106p Norsafe JYN100 Life boats(100% each side)

### Accommodation

100 persons,2 Day rooms,2 Client Cabins

Project meeting room

Conference room

Sky lobby/cinema

Gymnasium

Helideck reception

28 Single Cabin/38 Double Cabins

### Helicopter Deck

Supplier                                              Maritime Products AS

Size                                                                           D=26,1

Aircraft type                                                     S92 Sikorsky

Classification                     NMD,CAP437,MODU1989,DNV

### Fuel Consumptions

In Port                                                                 3 m³/24 hrs

Transit                                                               25 m³/24 hrs

Full Speed                                                        43 m³/24 hrs

DP                           7-15 m³/24 hrs(depend on the weather)

### Crane

Offshore Cranes

1×National Oilwell 250t Subsea AHC Knuckle Boom Crane.

Main winch max lifting capacities 250t,

96mm wire, length 2500m.

Operation radius 12-36m,

Aux winch max lifting capacities 20t,

34mm wire, length 2500m.

Operation radius 9-40m.

Deck Cranes

2×Electro-Hydraulic deck/provision cranes

Each crane capacity:3.0t/15m

Specifications                               DIV

DIV Diving Engineering Co., Ltd

ROV

### Schilling HD32 ROV

Vehicle

HPU                                                   150Hp system,112kW

Depth rating                                                               3000m

Vehicle dimensions                                           Length 2900

mm×Width 1700 mm×Height 1900mm

Speed                                                        Fwd/Aft 3.2knots

Lateral 2.3 knots

Vertical 2.5 knots

Bollard pull                                             Forward/Aft,900 Kg                                                                        Lateral,900Kg

Vertical(Up/Down),850Kg

Payload                                                                      250Kg

Manipulators                               7 function Schilling Titan 4

5 function Schilling Rigmaster

Auto function

Auto Heading/Depth/Altitude/Roll/Pitch

Stationkeep(DP)

Camera                                                           (5)Kongsberg

(1 HD color camera fiber output)

(1)TITAN 4 Wrist camera

Lights                                        (10)Variable 120VAC/250W

Pan & Tilt                                       2 Electric Pan & Tilt Unit

Valve Pack                                                        (2)8-function

TMS                                            With 900m tether Capacity

LARS                                       Active Heave Compensated

### Schilling HD05 ROV

Vehicle

HPU                                                   150Hp system,112kW

Depth rating                                                               3000m

Vehicle dimensions                                           Length 2500

mm×Width 1700 mm×Height 1900mm

Speed                                                        Fwd/Aft 3.0knots

Lateral 2.2 knots

Vertical 2.2 knots

Bollard pull                                                Forward,1000 Kg

Aft,880Kg , Lateral,930Kg

Vertical(Up),465Kg, Vertical(Down),576Kg

Payload                                                                      250Kg

Manipulators                               7 function Schilling Titan 4

5 function Schilling Rigmaster

Auto function

Auto Heading/Depth/Altitude/Roll/Pitch

Stationkeep(DP)

Camera                                                           (5)Kongsberg

(1 HD color camera fiber output)

(1)TITAN 4 Wrist camera

Lights                                        (10)Variable 120VAC/250W

Pan & Tilt                                       2 Electric Pan & Tilt Unit

Valve Pack                                                        (2)8-function

TMS                                           Schilling XE™ 900m tether

Capacity,with 425m tether

LARS                                       Active Heave Compensated