DIV

DIV Diving Engineering Co., Ltd

SAT8 180msw/9 man

Saturation Diving System

Specifications                               DIV

DIV Diving Engineering Co., Ltd

SAT8 180msw/9 man Saturation Diving  System

### MAIN PARTICULARS

Year built                                                                                 2014

Builder                                                                                       FDI

Class Society                                                           ABS(tank age)

Testing/Certification                                            IMCA D018/D024

Standard

### COMPONENTS

Weight     Dimensions (L*W*H)

Main Module with Diving Bell	    68t	       12192*1962*6400

Saturation Control Van		    7.5t	         3048*3048*7010

HRC		                    15t		3048*42672

HRC Control Van		    3t	         3048*2438*2590

Main Umbilical Basket		    3t	        2438*2438*1371

Emergency Umbilical		    1t	        1000*1000*2000

Stores/Workshop Container	    7t	        6096*2438*2438

Machinery Van		   10t	        3048*3048*7010

### SERVICES REQUIRED

Electric Power - System                                              440V, 60HZ

Electric Power – Hot water machine                           440V, 60HZ

Emergency generator                                     250kVA 440V,60HZ

Compressed Air                                                        400cfm,90psi

Seawater                                                                             25gpm

Fresh Water                                                                          5gpm

### LIVING ACCOMMODATION CHAMBERS

Main Living Chamber                                                           6 man

HRC M/Lock（9.943m³）                                                    3 man

### DIVING BELL

Capacity men                                                                              3

Diver Lockout Capability                                                              2

Diver Hat Mounted Camera Systems                                          2

Volume                                                                                 4.5 m³

Mating to DDC                                                                       Side

Buoyancy                                                                        Negative

### RECLAIM SYSTEM

Type                                                        Dive x Gasmizer System

Pumps                                                                                 Haskel

Bell Capacity                                                         2 Diver Reclaim

Dive Control System                                                             Divex

### HYPE RBARIC EVACUATION SYSTEM

Type                                                      Side-off/Float-off HRC

Launch System                                 Crane/winch deployment

Propulsion                                                                  Tow only

Capacity men                                                                          9

Control Van                                            Dedicated w/umbilical

### UMBILICAL

Main Bell                                                                    FDR 25m

Emergency Bell                                                 FDI built 180m

Lockout Diver1                                                                   60m

Lockout Diver2                                                                   60m

Bell men                                                                             65m

### LIFE SUPPORT MACHINERY

Diver Hot water Unit                                                    FDI Built

CUM Units                                                                         UTS

Scrubbers                                                                          Dive

Specifications                               DIV

DIV Diving Engineering Co., Ltd

HAI YANG SHI YOU 286

### GENERAL DESCRIPTION

Type                                  MP Subsea Construction Vessel

Design                                                                        SDARI

Accommodation                                                150 persons

Call Sign                                                                    BFBF6

IMO No                                                                   9662239

Flag                                                                             China

### Classification(DNV)

DNV+1A1,ICE-E,SPS,DPS3,CRANE,HELDK,BIS,COAT-PSPC(B),CLEAN,E0,NAUT-OSV(A),DK(+),COMF-V(3)C(3)CSA,Special Purpose Ship(2008),Ice Class B,DP-3,Lifting

Appliance, Helicopter  Facilities, PSPC

(B),In Water Survey

CSM,AUT-0,OMBO,Clean,COMF

(NOISE3),COMF(VIB3)

### Main Dimensions

Overall length                                                          140.75m

Length between p.p                                                127.85m

Breadth molded                                                          29.0m

Depth main deck                                                         12.8m

Draft(max)                                                                     8.5m

### Deck Equipment

2×Windlass/Mooring winches fwd.-MEP

2×Mooring winches aft- MEP

2×Provision cranes

### Deck Space

Deck Material                                                      Steel/wood

Moon pool                                                              7.2×7.2m

Main Deck Area                                                        1900/㎡

### DP System

DPIII                                 A redundant Dynamic Positioning

(DP)system

### NAV/Com

1×MF/HF SSB Simplex radio Telephony Station 150W

2×EPIRB Cospas/Sarsat free-float beacon

2×Radar transponders for life-raft/boat(9GHZ)

3×Portable waterproof VHF radios

Satellite communication, mobile telephones/fax

VHF/UHF and mobile telephones/fax/ VHF/UHF

Internal communication systems

1×complete PA system and 1×telephone system to be installed

Infrastructure/LAN

### Power & Propulsion

Main                                       6 sets,4×3884kW+2×2904kW

Em.Gen                                                                1×500 kW

### Accommodation

150 persons

28 Single Cabin/39 Double Cabins/11 cabins with 4 beds

### Thruster

Stern                      2×azimuth thrusters     Max.Continuous

Rating                                                       abt.4500kW each

Bow                                2×tunnel thrusters forward. Rating

Abt.                                                                          2200kW

1×Retractable azimuth thruster

Rating                                                                     2200kW

### Rescue

Life boat

2×fiberglass reinforce plastic, totally enclosed type lifeboats

1×life boat on each side, with capacity of 50%(75persons)

2×Fast Rescue Craft/Man Over Board Boat

2×davit launched type inflatable life rafts, with capacity of 20 persons

### Helicopter Deck

Diameter deck                                                             22.2m

Max. take-off-mass                                                       12.8t

Aircraft type                                                    Sikorsky S-92,

S-61 and Euro Copter EC225 Super Puma MKII

### Tank Capacities

Marine Gas oil/diesel oil, abt.                                  2800 m³

Potable water, abt.                                                    750 m³

Water ballast, abt.                                                   7300 m³

Rig chain lockers abt.                                              1000 m³

### Crane

Master Type Crane

SWL(2 falls)@ 16.5m                                                 400Te

SWL(1 falls)@ 31.5m                                                 200Te

Dynamic factor                                                                1.1

Working Range

Single line                                                             7.8-37.0m

2 falls                                                                    8.8-39.5m

Storage Capacity

Hook travel, single line                                               3000m

Main Hoist Wire Rope

Wire rope diameter                                                     92mm

Wire rope MBL                                                           650 Te

Specifications                               DIV

DIV Diving Engineering Co., Ltd

ROV

### Schilling UHD78/79 ROV

Vehicle

HPU                                                   200Hp system,149kW

Depth rating                                                               3000m

Vehicle dimensions

Length 3000mm×Width 1900mm×Height 2000mm

Speed                                                        Fwd/Aft 3.2knots

Lateral 3.2 knots

Vertical 2.5 knots

Bollard pull                                           Forward/Aft,1200 Kg

Lateral,1200Kg

Vertical,1000Kg

Payload                                                                      300Kg

Manipulators                               7 function Schilling Titan 4

5 function Schilling Rigmaster

Auto function

Auto Heading/Depth/Altitude/Roll/Pitch

Stationkeep(DP)

Camera                                                           (5)Kongsberg

(1 HD color camera fiber output)

(1)TITAN 4 Wrist camera

Lights                                        (12)Variable 120VAC/250W

Pan & Tilt                                       2 Electric Pan & Tilt Unit

Valve Pack                                                        (2)8-function

TMS                                           Schilling XE™ 900m tether Capacity,with 850m tether

LARS                                       Active Heave Compensated